توريد اجهزة حركة قوية accelerometer عدد 3 وجميع ملحقاته التشغيلية بما يتوافق مع المواصفات المرفقة : - Three sensor channels with an internal EpiSensor triaxial deck , scale range 1g , 2g , 4g - 24-bit/32 bit Delta converter, one per channel - Built-in GPS/GNSS and PTP timing with GPS high quality (length of cable 5 M at least - Record and communicate multiple sample rates - Data offloaded automatically to removable thumb drive Connected to the USB host port. Parallel recording (mirroring) Data on an external USB thumb drive - Support threshold trigger algorithm and STA/LTA trigger algorithm - State-of-health monitoring, including input and system Voltages, internal temperature, communication link diagnostics - Internal storage at least 30 GB - IP Security through SSH and SSL - Reverse voltage protection, self-resettable fuses and over/under voltage - System Status LEDs - Ethernet interface: Real Time Telemetry (Multiple destinations TCP/IP web server for parameter setup, event retrieval via FTP/SFTP; supports Contact (POC) name service. Modem: External, cellular or POTS, connected via the USB 2.0 Host - Consult factory for details. Protocols: Real-time data streaming via compatible server or via public SEED Link and Earthworm protocols - State-Of-Health: Input voltage, Time synchronization internal temperature - Data visualization: Waveform Viewer for continuous waveform display and File Viewer for triggered event display - Consumption: less than four W operational - Input Voltage: 12 VDC or 24 VDC - Mounting central bolt, three adjustable feet with bubble level - Temperature range: -10° to 60°C operational - Humidity: 0-100% RH (non-condensing) - Enclosure rating: IP67

تدريب عدد اثنين من الفنين على تركيب وتشغيل ومعالجة البيانات لمدة 4 ايام او بما يتوافق مع الشركة المصنعة .

مراعاة الانظمة المعمول بها في القطاع الحكومي من حيث مدة التورد والكفالات و الصيانة

مدة التوريد : 3 اشهر شامل فترة التدريب

مواصفات توريد اجهزة الحركة القوية وتدريب عدد 2 من الفنيين عليها :